Software Requirements for RIS/VTS/ER–SAR System

Overview

The software dashboard platform shall act as the central command and control system for Integrated Command and Control Centre in GOG  for managing vessels/ferries operating on the Brahmaputra River. It shall integrate data from multiple sources — including onboard hardware devices, weather and river sensors, and external agencies (ASDMA, IMD, IWAI, etc.) — to provide a real-time situational awareness, monitoring, and emergency response system using the GSM (2G,3G or 4G).

The software shall support both web-based and mobile interfaces and be hosted on AIWTDS’s secure data centre where the ICCC had the dashboard web app while the vessel/ferry operator and the ER and SAR teams have the mobile applications installed.

Core Functional Modules

| Module | Description |
| --- | --- |
| Realtime Vessel Tracking & Monitoring | - Real-time tracking of boats/ferries through GPS data received from onboard units using GSM. - Display vessel position, heading, and speed on digital river maps. - Maintain vessel identity database (Boat ID, owner details, crew info). -Raise alarms when ever there is a breach in the route by any of the monitored vessel or when there are chances of collision. -Generate heat maps with help of IOT devices on foggy days for Monitoring purpose. |
| Emergency Communication & Alert Management | - Two-way message exchange between ICCC and vessels using GSM over SIM based protocol. - Display and log emergency alerts, boundary-crossing alerts, SOS triggers, and weather advisories. - Support both broadcast (to all vessels) and targeted (to specific vessels/zones) messages. |
| Geo-fencing & Boundary Violation Detection for the Terminal Safety Management | - Configure virtual zones (geo-fences) along river stretches. - Auto-generate alerts when vessels enter or exit predefined zones. - Allow authorized users to dynamically create or modify zones. -Auto generate alarm when ever there is a breach by any vessel with respect to any Geo Fenced boundary. |
| Incident Reporting and Documentation (Emergency Response & SAR (Search and Rescue) Management) | - Instant SOS alert reception with vessel coordinates and contact details. - Incident tracking dashboard for rescue operations. - Integration with ASDMA and emergency response agencies for coordinated action. -Data about the incident flows flawlessly between the SAR team and the vessel through the mobile application and is stored in the central server as a disaster record for data analytics and future course for prevention. |
| Weather & River Data Integration for the Early Warning Assistance | - Integration APIs with ASDMA data sources for river levels, rainfall, and flood alerts. - Display current and forecast weather conditions per river stretch. - Configurable rule engine for automatic warnings (e.g., “High Flood Risk Zone – Activate Alert”). |
| Data Analytics & Reporting | - Generate automated reports on vessel movements, alert frequency, and response times. - Dashboard visualization using charts, heatmaps, and timelines. - Export in PDF/Excel formats. -SOS data analysis and storage for future reference. |
| System Administration & User Management | - Role-based access control (Admin, Operator, Viewer). - SAR team access with viewer and operator access in case of incidents. - Audit trail for user activity. - Configurable permissions for various stakeholder departments. |
| Bathymetric data for Navigational Assistance | Incorporating bathymetric data in the system to provide navigational route to the boat pilots. These routes can be used as a GEO Fence to the vessel operators. These predict the navigational paths for the vessels according to the Data provided. |
| Integration & API Management | - RESTful APIs for data exchange with onboard devices and third-party systems (e.g., SNI survey database, ASDMA). - Secure API gateway with authentication and encryption. - Support for MQTT/TCP-based message brokers for real-time data ingestion with help of GSM SIM based Protocol. |
| Risk Management | -Identify Risks by the users of the application -Data Patterns from the Data collected to Mitigate the Risks Identified -Realtime decision making by the users from prior experience or Data Patterns. |  |

Platform Architecture

| Component | Specification |
| --- | --- |
| Architecture Type | Modular, microservices-based architecture supporting scalability and high availability. |
| Hosting Environment | SDC |
| Frontend | Web application (responsive), accessible on standard browsers; optional Android app for field users which is Progressive Web Application |
| Middle Tier | Developed using open-source technologies (e.g., Node.js, Python, Java, C# , or equivalent). |
| Database | Relational or time-series database supporting geospatial data (PostgreSQL/MySQL/MongoDB with spatial extensions). |
| Map Integration | Integration with OpenStreetMap, Bhuvan, or equivalent GIS platform with offline caching. |
| Real-time Data Layer | WebSocket/MQTT-based data streaming for live tracking and alerts over GSM. |
| Security | Role-based access, HTTPS encryption, audit logs, and data integrity checks. CERT-IN based rules and protocols followed. |
| Failover & Redundancy | Automatic failover support for server and database layers With DMR sites and load balancers. |
| Data Backup | Automated daily backup with 15-day retention; restore capability. |

4. Integration Interfaces

The software must integrate the following external data sources and systems:

| Agency/System | Data Type | Integration Method |
| --- | --- | --- |
| ASDMA | Weather alerts, rainfall, flood data, river level | API |
| IWAI / WRD Assam | Bathymetric, and navigation data | Binary file to route data |
| SNI Portal | Vessel survey data and owner details | Excel file to Database |
| Onboard Devices | Position, SOS, alerts, event data | Push notifications using GSM on SIM based protocols. |
| ICCC Dashboard | Live monitoring and analytics | Web-based visualization |

Key Performance Requirements

| Parameter | Requirement |
| --- | --- |
| System Uptime | ≥ 99% during operational hours |
| Response Time | < 2 seconds for alert/event display |
| Data Refresh Rate | ≤ 30 seconds for live vessel updates |
| Scalability | Support minimum 1,000 vessels concurrently |
| Alert Latency | ≤ 10 seconds from event trigger to display |
| Data Retention | Minimum 5 years archival with retrieval capability |

User Interfaces

- ICCC Dashboard: Central monitoring interface with layered maps and alert panels.

- Mobile Application (Optional): For field personnel/boat pilots.

- Analytics Portal: Role-based access for management and reporting users.

Security & Compliance

- Compliance with ISO 27001, MeitY Security Guidelines, CERT-IN and Govt. of India data policies.

- Data encryption in transit and at rest.

- Regular vulnerability assessments and penetration testing (VAPT).

Documentation & Training

- Complete user and technical manuals.

- API documentation for integration.

- Onsite/virtual training for ICCC operators and technical staff

Maintenance & Support

- 24×7 technical support during operational phase.

- Software updates and bug fixes included for 3 years post Go-Live.

- SLA-based response and resolution times.

Ownership & Intellectual Property

All source code, APIs, data models, and configuration files developed under this project shall remain the property of AIWTDS.

The vendor shall hand over all credentials, code repositories, and documentation upon project completion.

Deliverables

- Web-based ICCC software (live dashboard)

- Mobile app for the boat operators and the SAR Team

- Integration APIs for hardware and external systems

- Reports and analytics module

- Documentation and training manuals

Indicative Dataset Framework for River Information System (RIS)/Vessel Tracking System (VTS)

Hydrographic & River Morphology Data

| Dataset Name | Description | Source Agency | Update Frequency | Usage in RIS/VTS |
| --- | --- | --- | --- | --- |
| Bathymetric Data | 3D riverbed depth profile (x, y, z coordinates i.e. Latitude, Longitude and Depth) | AIWTDS | Monthly / Quarterly | Navigation route plotting, depth contour visualization |
| Water Level Data | Real-time water level at various ghats and stretches | ASDMA | Hourly / Daily | Dynamic depth estimation, route advisories |
| Flow Velocity & Direction | Current speed and direction data | WRD/ CWC / IWAI sensors | Daily / Real-time (if sensors available) | Navigation route optimization |
| River Cross-section Profiles | Width, depth, and slope data at specific points | IWAI /CWC/ Survey agencies | Static (annual updates) | River modelling and design reference |
| Sedimentation / Siltation Data | Areas of high silt accumulation | WRD /CWC/ IWAI | Annual / Biannual | Route maintenance planning |

Navigational & Vessel Traffic Data

| Dataset Name | Description | Source Agency | Update Frequency | Usage in RIS/VTS |
| --- | --- | --- | --- | --- |
| Vessel Position Data | Real-time GPS coordinates of vessels | Onboard Devices (OBU) | Continuous (10–30 sec interval) | Live tracking, geo-fencing, replay |
| Vessel Master Data | Boat registration, owner details, capacity, engine type, etc. | SNI / AIWTDS | Static / Annual | Vessel identification and classification |
| Voyage Logs | Route history, docking times | Onboard Device / ICCC | Per trip | Performance analysis and auditing |
| Geo-fence Boundaries | Predefined navigation zones | ICCC Config | Dynamic | Boundary crossing alerts |

Meteorological & Environmental Data

| Dataset Name | Description | Source Agency | Update Frequency | Usage in RIS/VTS |
| --- | --- | --- | --- | --- |
| Weather Forecast Data | Temperature, humidity, wind speed/direction | ASDMA | Hourly / 6-hourly | Route advisories and alerts |
| Rainfall Data | Precipitation levels by location | ASDMA | Daily / Hourly | Flood prediction and route closures |
| Flood Alerts / Early Warning | Flood risk zones, alerts | NRSECWC | Event-driven | Emergency warnings |
| Visibility / Fog Data | Fog density and visibility levels | Sensors | Hourly / Real-time | Navigation safety advisories |
| Water Temperature & Turbidity | Measured at sensors or survey stations | WRD / IWAI | Daily / Weekly | Environmental assessment |

Emergency, Safety & Incident Data

| Dataset Name | Description | Source Agency | Update Frequency | Usage in RIS/VTS |
| --- | --- | --- | --- | --- |
| SOS / Distress Alerts | Real-time emergency alerts from vessels | OBUs / ICCC | Event-driven | Rescue coordination |
| SAR Team Deployment Data | Locations of emergency response teams, boats, equipment | ASDMA / AIWTDS | Real-time / Daily | Rescue management |
| Incident Reports | Logs of past accidents, causes, response details | AIWTDS / ASDMA | On occurrence | Risk analysis, pattern recognition |
| Fire / Collision / Obstruction Reports | Field data on navigational hazards | Field Teams / Mobile App | Event-driven | Real-time alerts |
| Emergency Contact Database | Contact info of local rescue and health authorities | AIWTDS / ASDMA | Annual / On change | Coordination readiness |

Regulatory, Operational & Administrative Data

| Dataset Name | Description | Source Agency | Update Frequency | Usage in RIS/VTS |
| --- | --- | --- | --- | --- |
| License & Certification Data | Vessel, pilot, and operator licenses | Transport Dept. / AIWTDS | Annual | Compliance verification |
| Operational Permissions / Routes | Approved ferry routes and schedules | AIWTDS | Static / Periodic | Route validation |
| Inspection & Maintenance Logs | Vessel inspection records | SNI / AIWTDS | Periodic | Compliance monitoring |
| Survey Reports | Digital copies of vessel and river surveys | SNI | As available | Reference for analytics and integration |

System-Generated & Analytical Datasets

| Dataset Name | Description | Generated By | Usage in RIS/VTS |
| --- | --- | --- | --- |
| Alert Statistics | Frequency and types of alerts | ICCC | Trend analysis |
| Device Health Logs | Hardware connectivity and uptime | ICCC Monitoring System | Maintenance planning |
| User Activity Logs | Operator usage, login details | ICCC | Audit trail |
| Historical Playback Data | Vessel movement replay | Database Engine | Training and investigation |

Data Formats and Standards

| Category | Preferred Format / Protocol | Remarks |
| --- | --- | --- |
| Real-time tracking | GPS data with Latitude and Longitude in JSON format | For vessel-to-ICCC communication |
| Static datasets | Riverine data from IWT on Digital Terrain with one- or two-dimensional data | For maps and boundaries |
| Bathymetric data | XYZ, S-57 / S-101 standards | For electronic navigation charts |
| Weather and early alerts | API Integration in JSON | For warning to prevent disaster with help of API integration |
| Reports and analytics | PDF / Excel / JSON exports | For user interface reports |

Data Ownership and Access Policy

- All datasets collected, generated, or integrated under this RIS project shall be owned by AIWTDS.

- Access to external datasets (e.g., ASDMA, IMD, IWAI) shall be governed by MOU’s or data-sharing agreements.

- The system shall support API-based interoperability for future extensions.